Distributed Control and Automation Framework Hands-On Session

This hands-on covers the basics of implementing an application in the Distributed Control and Automation Framework, including using an existing module and developing a new control module. It doesn’t cover development of a new generic I/O or processing module. For this hands on, the framework downloads, and additional documentation, visit the Reference Designs for Distributed Control Systems portal on ni.com/referencedesigns.

## Set-up (Already configured on Alliance Day machines)

Navigate to https://decibel.ni.com/content/groups/dcaf?view=documents and download the most recent version available (.vipc file). This package can be installed to any LabVIEW version from 2013 to present.

# Exercise 1: Configuring a Simple Temperature Control Application

Introduction

This exercise demonstrates the implementation of a simple temperature chamber controller application using the Distributed Control and Automation Framework (DCAF). This exercise makes use of a model of the chamber to simulate its I/O and allows users to define the setpoint and PID gains of the control algorithm through a simple user interface.

During the exercise you will learn to identify how inputs and outputs from different modules are mapped into the DCAF Tag Bus to provide a synchronized communication between modules.  You will also learn how to create a UI and map it to the Tag Bus.

Our Temperature Simulated Controller will consist of 2 DCAF engines: the UI and the Temperature Controller Simulation.

Figure 1.1

This Simulation Engine is implemented in such a way that it will be easy to migrate from a simulation to a real time controller by adding a cRIO target and moving the Simulation Engine below it.

Notice the different components of each engine listed in Fig 1.1. Tags, Mappings and UDP repeat in both engines. The Temperature Controller Logic and the Temperature Chamber Model are Modules created specifically for the Simulation Engine. Similarly, the UI Reference Module is only present in the UI Engine. Here is a brief description of each of these components.

Tags: Tags are scalar variables that can be mapped as inputs or outputs in any module within a DCAF engine. This pane allows the user to add or remove tags to the engine and configure tag properties such as Tag Name, Data Type, Default Value, and Description.

Mappings: This pane allows the user to connect tags as inputs or outputs in DCAF modules. For example, an Output Channel from the Temperature Controller Logic Module can be mapped into a Tag which can be mapped as an Input Channel to the Temperature Chamber Model Module.

UDP: This pane is also present in both components. It is designed to share tags between Engines by mapping each tag as an Engine Input or Output. All the tags that are intended to be shared between engines need to be defined in the Tags Pane of each engine with the same names.

UI Reference: This module only works in the UI Engine. It provides a front panel with the necessary controls and indicators to allow the user interact with the DCAF Engine.

Temperature Controller Logic: This is a custom DCAF Model designed to provide a Temperature Control Logic. If the Simulation Engine is moved to a cRIO Target and the Temperature Chamber Model is replaced with real IO, this module could remain the same.

Temperature Chamber Model: This module provides a simulated model of a Temperature Chamber. This module could be replaced or overwritten to eventually provide IO from a real Temperature Chamber.

Part 1: Project Creation and UI

During this first part of the exercise you will be guided through the process of creating a DCAF project from scratch using a template and learning how to add a User Interface to your DCAF project.

- In LabVIEW go to File >> Create Project.. and select TBDF. From the displayed list select Basic Execution Template and press the Next button.

- Name the project Temperature Controller and select \\Desktop\DCAF Hands On\Exercises\Temperature Controller\Runtime\Temperature Controller as the Project Root. Type TCRL as the File Name Prefix.

- Verify your project window looks like Figure 1.2.

Figure 1.2

- Add to the project a Configuration File with the Engines for the Temperature Controller partially configured and mapped. Under My Computer add SimulatedSystem.pcfg located at \\Desktop\DCAF Hands On\Exercises\ Temperature Controller.

- To speed up the exercise, a UI has already been created. Under My Computer, add TCRL User Interface.vi to the project located at \\Temperature Controller\Runtime.

- Take a look at the names of the Labels in the Block Diagram. This is important to correctly map the tags to the UI. Controls and Indicators will be directly updated through the DCAF UI Engine, so there is no need to add more code in this VI.

Figure 1.3

Save and Close TCRL User Interface.vi.

- Open TCRL Host Main.vi Block Diagram. Drag and Drop TCRL User Interface.vi into the Block Diagram from the Project Window. Force TCRL User Interface.vi to execute in parallel to the DCAF engine connecting as appears in Figure 1.4.

Figure 1.4

- Open TCRL Host Main.vi Front Panel. In the configuration file path control browse for SymulatedSystem.pcfg located at \\Temperature Controller.  Select this as default value for this control. Save and Close this VI.

Part 2: Adding Required Classes

DCAF has been developed using LabVIEW Object Oriented Programming. Therefore, the code will only run if the classes used within a specific configuration are added to the project. DCAF provides a Script Engine that will help you with this every time you add or remove elements to your engines in the Configuration Editor. This is not automatic, so you have to remember to run this scripting tool when you make this kind of changes in your DCAF Configuration.

- Open the Standard Configuration Editor for TBD by navigating in LabVIEW to Tools>>TBDF>>Launch Standard Configuration Editor…

- Navigate within the editor to Tools>>Edit Plugin Search Paths.

- Add a search path to the TBM plugins for this example located at \\Temperature Controller if it’s not already there.

- In the DCAF Configuration Editor go to File>>Open and search for the SimulatedSystem.pcfg Configuration File located at \\Temperature Controller.

- Take a couple of minutes to go through each component in the Simulation and UI Engines.

- Open the Temperature Controller project located at \\Temperature Controller\Runtime if not already opened.

- Open TCRL Host Module Includes.vi and verify the Block Diagram is empty. This VI will load the required classes when TCRL Host Main.vi executes. Use the Scripting Tool to add the corresponding classes to TCRL Host Module Includes.vi. To use the Scripting Tool go back to the Configuration Editor and select Tools >> Script Include VI. Select TCRL Host Module Includes.vi located at \\Temperature Controller\Runtime. Select PC when the Enter Target to Script window appears. Verify the corresponding classes had been added to TCRL Host Module Includes.vi and compare them to Figure 1.5:

Figure 1.5

- Save and close TCRL Host Module Includes.vi.

Part 3: Mapping Tags in the Configuration Editor

This DCAF project has 2 engines: The Simulation Engine and the UI Engine. Both engines have listed Tags, Mappings, and UDP items. The rest are specific modules for each engine.

- The Tags item refers to the list of tags in the Tag Bus for each engine.

- The Mappings item allows to configure and visualize the connections between each Module parameter (Input/Output) and the Tag Bus.

- The UDP item publishes tags that can be shared with another engine that might be in the same target or in a different one.

During this last part of the exercise you will learn how to map tags between modules through the Tag Bus in each engine and share tags between engines through UDP.

- Open the Standard Configuration Editor for TBD by navigating in LabVIEW to Tools>>TBDF>>Launch Standard Configuration Editor…

- We will first map our UI to the UI Engine Tags. Beneath the UI Standard Engine select UI Reference. Notice the table in the Static Configuration tab is empty. Press the Browse button next to the UI to Load textbox. Browse for TCRL User Interface.vi located at \\Temperature Controller\Runtime.

- Press the Configure from UI button. When the pop up asking to Automatically map tags to channels appears select Yes. Verify your mappings comparing them with Figure 1.6.

Figure 1.6

- Save the changes in the Configuration Editor.

- Open and run TCRL Host Main.vi. Try changing the Setpoint and the other controls. Do you see any change in the temperature value displayed in the Graph?

Figure 1.7

You shouldn’t see any change in the signal since we only connected the tags in the UI Engine. There are still some tags in the Simulation Engine that we need to map so we can see the PID standard behavior.

- Stop the TCRL Host Main.vi and return to the Configuration Editor. We will review the connections in each component on both engines to understand the tag dataflow and connect the tags that are missing to make it run.

- We will start with the Simulation Engine. First select the Tags component and take a look to the tags.

Figure 1.8

- This tags are used as inputs or outputs in the rest of Simulation Engine modules: Temperature Controller Logic, Temperature Chamber Model and UDP. Notice all of them are Doubles except for Fan on?.

- Go to Mappings and select the Manual Mapping tab. This section will allow you to have a better look of the tag flow in this application. In the left pane you will see all the channels that haven’t been mapped. Just take a look, don’t make changes.

Figure 1.9

- Go to the Temperature Controller Logic Module. Notice there are two variables that don’t appear in the Tag list: output range high and output range low. This are internal variables with constant values defined statically. The rest should be mapped to a tag.

- The last 3 channels should be connected to a tag (Fan on?, fan, and lamp). To connect a channel to a tag, take the cursor to the corresponding cell in the Mapped to System Tag column, right click, and select the corresponding tag from the Available Tags list.

Figure 1.10

- Verify your table looks like the following image:

Figure 1.11

- Before going to the next module notice the Direction column. Processing parameters are module inputs while processing results are module outputs. Some of the processing parameters in this module come from the UI Engine and others come from the Temperature Controller Logic Module. The two processing results in this module will go through the Tag Bus as inputs in the Temperature Chamber Model Module.

- Go to the Temperature Chamber Model Module. Notice all the channels are disconnected from any tag. The only disconnected channel should be Ambient Temperature.  Fan PWM and Lamp PWM channels are processing parameters in this module that should come from the Temperature Controller Logic Module. Thermocouple Reading is a processing result that should be used as the feedback signal in the Temperature Controller Logic Module and will also be sent to the UI Engine to be displayed in the graph. Following the same instructions as in step 11, map Fan PWM, Lamp PWM, and Thermocouple Reading channels to Fan, Lamp, and Thermocouple tags. Verify your table looks like the following image:

Figure 1.12

- Go to the UDP Module in the Simulation Engine. Go to the Channel Mapping Tab. Notice the tags in the From External Engine (Inputs) and To External Engine (Outputs) boxes. Notice the Fan tag is still as an Available Tag. There is no need to move it since it is not needed in the UI Engine, it is only used internally in the Simulation Engine.

Figure 1.13

- Go back to Mappings in the Simulation Engine. Notice now there are only 3 channels that haven’t been mapped. There are no tags for those channels since they are configured statically in their corresponding modules or set as default. All the channels that originally were unmapped now appear mapped in the right pane. Take some time to review the mapping directions to have a better understanding of the data flow.

Figure 1.14

- Now you have understood how the Simulation Engine works, step into the different components of the UI Engine to understand how it interacts with the Simulation Engine. Notice that the Inputs for the UI Engine UDP Module are the Outputs for the Simulation Engine UDP Module and vice versa.

Simulation Engine UDP				UI Engine UDP

Figure 1.15

- To better understand the dataflow please take a look to the diagram in Figure 1.16.

Figure 1.16

- Go to File >> Save and close the Configuration Editor.

- Open the Temperature Controller Example Project if not already open. Open and run Host Main.vi.

- Modify the Setpoint and the other controls in the UI. You should now see the temperature being controlled by the Simulation Engine.

Figure 1.17

End of Exercise 1

# Exercise 2: Adding Standard Modules to the Temperature Control Application (TDMS & CVT)

In Exercise 1 you developed a Simple Temperature Control Application using DCAF. We are going to add standard features such as TDMS and CVT to learn how to add standard DCAF modules to your application.

Part 1: Add TDMS

Adding TDMS is a specific module that might become really handy in a DCAF application. This part of the exercise will guide you through the process of adding TDMS logging to your DCAF application.

- Open the Temperature Controller project you developed in Exercise 1 if not already opened.

- Open the Configuration Editor and load SimulatedSystem.pcfg if not already opened.

- Right click the Simulation Engine and select Add>>Other>>TDMS datalogger as shown in Figure 2.1.

Figure 2.1

- Select the TDMS datalogger item you just created. In the Static Configuration tab move Temperature, Setpoint, P, I, and D to the Configured to the Configured to Log box.

Figure 2.2

- Go to the Datalogger Configuration tab. Press the first browse button to select a File path. Browse to \\Temperature Controller\Runtime and create a folder named Data. Type Temperature Measurements as the File name.

- Press the second browse button to select a Historical Directory. Browse to \\Temperature Controller\Runtime and create a folder named Historical. Type Temperature Measurements as the File name. Verify your file paths with Figure 2.3.

Figure 2.3

- Since we added a new module, loaded classes should be updated. Use the scripting tool explained in Exercise 1>>Part 2>>Step 7 to update the classes in TCRL Host Module Includes.vi. Verify the class has been successfully added to TCRL Host Module Includes.vi.

Figure 2.4

- Run TCRL Host Main.vi. Do some changes to the setpoint and verify it still working and stop the VI.

- Go to the Historical folder you created located at \\Temperature Controller\Runtime and open the TDMS file just created. Verify the tags you added in the TDMS datalogger modules appear in the file and generated data.

Figure 2.5

Part 2: Add CVT

Another nice feature that can be added to your DCAF project is CVT. Sometimes you will need to share tags with code that might run asynchronously in parallel with the DCAF engine. CVT provides a way to communicate the DCAF engine with the rest of the code.

During this part of the exercise we will publish the Temperature tag and visualize it in the Front Panel of your TCRL Host Main.vi.

- Open the Temperature Controller project if not already opened.

- Open the Configuration Editor and load SimulatedSystem.pcfg if not already opened.

- Add a CVT configuration module to the Simulation Engine in the same way you added the TDMS datalogger.

- Select the CVT configuration module. Select To CVT direction. Move Temperature to the To CVT box. Save your configuration.

- Open TCRL Host Main.vi Block Diagram. Add a Read VI from the Current Value Table Function Palette. By default it is a double. Connect a string constant to the Tag Name Terminal and type Temperature.

Figure 2.6

- Finish the code as shown in Figure 2.7

Figure 2.7

- Rearrange front panel Indicators to get the new indicators into a visible area as shown in Figure 2.8.

Figure 2.8

- Save the changes in TCRL Host Main.vi. Go back to the Configuration Editor and update the classes for TCRL Host Module Includes.vi as you did for the TDMS Datalogger Module. Verify the CVT class is added to TCRL Host Module Includes.vi.

Figure 2.9

- Run TCRL Host Main.vi. The UI should still be working. Verify the value displayed in the new Temperature indicator in TCRL Host Main.vi corresponds to the value displayed in the Temperature chart in TCRL User Interface.vi

Figure 2.10

- Stop and close TCRL Host Main.vi.

- Take a look to the following diagram to verify the updated mappings.

Figure 2.11

End of Exercise 2

# Exercise 3: Creating a Custom Module

In this exercise we will implement a custom control algorithm for our temperature controller. This can be actually custom, or a simple PID controller as is used by the original example.

- Navigate to Project >> Create Project…

- In the tree on the left, select TBDF >> Modules and select User Control Module.

- Type Custom Temperature Controller as the Module Name.

- Look for the Project Root \\Temperature Controller\Module and create a folder called Custom Temperature Controller. Get into that folder and press Current Folder.

- Enter a new name for your module and select an appropriate project path.

- Add the following parameters to your new module as Channels. Channels represent data passed to or from your module during different execution stages, and channel names are case sensitive. The direction specifies whether the data is to the module or from the module, and is divided into inputs (data provided by input.vi), outputs (data provided to output.vi), and processing parameters and results (data passed to and from process.vi). For this hands on we will implement a processing step.

| Name | Type | Direction |
| --- | --- | --- |
| Temperature | Double | Processing parameter |
| setpoint | Double | Processing parameter |
| output range high | Double | Processing parameter |
| output range low | Double | Processing parameter |
| Kc | Double | Processing parameter |
| Ti | Double | Processing parameter |
| Td | Double | Processing parameter |
| Fan on? | Boolean | Processing parameter |
| fan | Double | Processing result |
| lamp | Double | Processing result |

Your configuration should look like the one shown in Figure 3.1.

Figure 3.1

- Press Finish.

- Your new project will appear. Navigate to Custom Controller Module runtime.lvclass and open user process.vi. This method should have two clusters, one input and one output, which match the list of tags above. This clusters were created based on the table you just filled in.

Figure 3.2

- Open process.vi in the overrides folder.

Figure 3.3

On either side, Scripted Accessors convert tag bus data into your user-defined cluster. These methods are not automatically generated by the project scripting tool and must be generated when the project is first scripted or after any change to the interface. If you need to make changes to the interface, go to Appendix A.
Note: During this process, the lvclass files are modified and so LabVIEW requires that your classes are not loaded in multiple contexts. That is, you must close any other projects which currently have the class loaded. If you see a “lock” icon over the class, the script will not work. To script these methods:

  - Open Tools >> TBDF >> Launch Control Module Scripting Utility…

  - Drag the runtime class from the project over the runtime class path control or browse for it manually, then do the same thing for the configuration class (YourModuleName Configuration.lvclass).

  - Because you used the script, you can leave most options as the default and press Run.

- The VIs that were scripted should look like the ones in Figure 3.4.

Figure 3.4

- Return to process.vi. User process.vi is the function being called in the middle and is the function you will implement. You will implement the same PID control that the Temperature Controller Module used in previous exercises. To match the behavior you will follow the next steps:

  - Drop down an instance of PID.vi from Control and Simulation >> PID >> PID.vi.

  - Drop down the following functions: 1 Unbundle by Name, 2 Bundle functions, 1 Bundle by Name function, and a Select function.

  - Wire the Unbundle by Name function to the Parameters to process cluster. Expand all the terminals.

  - Wire Parameters to process.Temperature to PID.vi process variable input.

  - Wire Parameters to process.setpoint to PID.vi setpoint input.

  - Bundle Parameters to process.output range high and Parameters to process.output range low and wire the cluster to PID.vi output range input

  - Bundle Parameters to process.Kc, Parameters to process.Ti, and Parameters to process.Td and wire the cluster to PID.vi PID gains input

  - Create a constant from Results from processing indicator and connect it to the Bundle by Name input cluster terminal. Expand the Unbundle by Name Function to have 2 terminals

  - Wire Parameters to process.Fan on? to the Select function and set the two values as t=100 and f=0. Wire the output of this function to the Results from processing.fan value

  - Wire PID.vi output to Results from processing.lamp.

  - The result should look something like Figure 3.5:

Figure 3.5

- Save the new project and close it.

- Reopen Temperature Controller Example.lvproj if you closed it, and reload the standard configuration editor (Open Tools >> TBDF >> Launch Control Module Scripting Utility…).

- Navigate to Tools >> Edit Plugin Search Paths.

- Press Add and navigate to the location of your new control module.

- Reopen <LabVIEW 2015>\examples\TBD Examples\RT Temperature Controller\SimulatedSystem.pcfg and then to go File >> Save As… to make a copy of the configuration. For simplicity, save it in the same location but call it SimulatedSystemCustomControl.pcfg.

- Now, right click on Standard Engine (Simulation) and select Add >> YourControllerModule. Then, select this new module from the tree.

- For each processing parameter tag, right click on the column “Mapped to System Tag” and configure the channel to be mapped to the appropriate system tag. You can look at “Temperature Controller Logic” to identify the correct mapping, which looks like this:

- Now, right click on the output ranges and press Set Default for each. Set output range high to 100 and output range low to 0 (this is not required, as the default if not set is 0). These are unmapped parameters, meaning they can be modified in the editor but they are constants at runtime – output range high will always be 100.

- Finally, we need to map our results to system tags. However, the editor prohibits mapping multiple writer channels (processing result or output) to a single system tag – mapping two writers to a single tag would introduce a race condition. So the next step is to either remove the mappings from the Temperature Controller Logic module, or to delete the module entirely. In either case the write reservations will be released. Reader channels may be ignored, as any tag can be read from multiple channels.

  - Select Temperature Controller Logic, right click on rightmost column for the “fan” and “lamp” channels, and select disconnect.

  - And/Or: Right click on Temperature Controller Logic in the tree and select Remove.

- Return to your module’s configuration screen and map channel “fan” to tag “Fan” and channel “lamp” to tag “Lamp”.

- Select to File >> Save.

- Open Host Main.vi (our top-level application VI) and navigate to Host Module Includes.vi on the diagram.  This function ensures that all appropriate modules are loaded into memory. You can also load precompiled modules (llb or lvlibp files) from disk, but for our purposes we will simply hardcode the appropriate modules.

- Drag an instance of YourModuleName Runtime.lvclass from the project onto the diagram of Host Module Includes.vi. This ensures that your new module is always loaded into memory.

- Now save and close this VI and return to Host Main.vi. On the front panel, enter the name of your configuration file (we suggested SimulatedSystemCustomControl.pcfg) in the file name control.

- Press run.

- The behavior should match that of the original controller if you used the same code. 
Debugging Note: While process.vi is a shared reentrant method and is therefore difficult to debug, user process.vi defaults to being non-reentrant and debuggable.

End of Exercise 3

# Appendix A: Modifying control module data

- If you made a mistake on your module or would simply like to add or remove different methods (for example, you want to add input functionality to your module), you will need to navigate to get data definition.vi in your runtime class. The “valid?” Boolean for each case of the case structure indicates whether or not the data interface is valid and if that method should be run. Setting a given Boolean to true tells the framework that you’d like to configure and run that method. If you followed the steps above, the Boolean should be true for processing parameters and processing results. The script will use these Booleans to determine if a given cluster is valid (the “placeholder” fields are just there to ensure the clusters are not broken).

- To add additional input or output channels, simply drop the appropriate controls or indicators into one of the four clusters in the user data virtual folder.